# Public Health Laboratory Risk Assessment Template:

Ebola Disease Testing

# Introduction

This template is designed to assist laboratories in the development of their risk assessment for the testing of orthoebolaviruses. It may not be an all-encompassing plan, as each facility will have laboratory-specific risk assessment procedures.

Ebola disease is caused by a group of viruses, known as orthoebolaviruses (formerly ebolavirus) which can cause serious illness and, without treatment, can result in death. There are four types of orthoebolaviruses that cause illness in people:

- Ebola virus (species Orthoebolavirus zairense) causes Ebola virus disease

- Sudan virus (species Orthoebolavirus sudanense) causes Sudan virus disease

- Taï Forest virus (species Orthoebolavirus taiense) causes Taï Forest virus disease

- Bundibugyo virus (species Orthoebolavirus bundibugyoense) causes Bundibugyo virus disease

Two other types of orthoebolaviruses have not affected people, to date:

- Reston virus (species Orthoebolavirus restonense) causes disease in non-human primates, like macaques

- Bombali virus (species Orthoebolavirus bombaliense) was recently identified in bats

Standard precautions have been highly effective in preventing transmission of bloodborne infection while handling blood and other potentially infectious material in the clinical laboratory. Standard precautions should be effective in preventing the transmission of orthoebolaviruses and other viral hemorrhagic fever agents in the clinical laboratory. However, orthoebolaviruses are high-consequence pathogens, and there has been limited experience handling specimens potentially contaminated with such a high-consequence pathogen in a clinical laboratory using current specimen handling procedures and automated instrumentation. Therefore, this risk assessment can help enhance precautions and personal protective equipment (PPE) when handling specimens from patients who may be at risk of having Ebola disease.

For additional information on reporting or identifying a select agent or toxin, please reference the FAQ: Report of Identification of Select Agent or Toxin and CDC’s Viral Hemorrhagic Fever Clinical Specimen Packaging and Shipping Guidance.

Visit the APHL Preparedness Planning and Training Resources for additional preparedness resources, including a Clinical Laboratory version of this template.

# Assessment Information

| Laboratory Unit/Section |  |
| --- | --- |
| Date of Assessment |  |
| Name of Assessor |  |
| Name of Organism/Agent | Orthoebolaviruses e.g., Orthoebolavirus zairense |

# Ebola Disease Testing Overview

| # | Procedure | Potential Hazard(s) | Control/Protection | Additional Information |
| --- | --- | --- | --- | --- |
| 1 | Package receipt and transfer to testing area | Leaking package | Transfer and open inside a Biological Safety Cabinet (BSC) PPE: nitrile/latex gloves, lab coat, safety glasses | Contact Responsible Official (RO)/Alternate Responsible Official (ARO)/safety officer immediately Disinfect exterior of sealed plastic bag prior to transfer to testing area If leaking occurs, properly doff contaminated PPE and wash hands before donning new PPE |
|  |  | Unexpected delivery | Immediately transfer to BSC and contact RO/ARO/safety officer of unexpected delivery If transporting specimen to another facility, use Category A packaging | Do not conduct testing without CDC authorization Notify key staff of expected package delivery ARO/RO notifies section/unit All Category A packages should be opened in certified class II BSC using safety blades In case of possible exposure, contact CDC Emergency Operations Center for clinical consultation (770.488.7100) Select agents and toxins should be classified as outlined by the US Department of Transportation (DOT) and packaged in accordance with the DOT’s Hazardous Materials Regulations |
| 2 | Transport of specimens between testing areas | Breakage of the specimen container | Specimens should be transported in a clearly labeled, durable, leak-proof transport container directly to the specimen handling area of the laboratory Container should have absorbent material at the bottom | Utilize a laboratory cart to transport the specimen container to keep your hands free and available to open doors, etc. Decontaminate exterior of transport container and cart after use |
| 3 | Don PPE with a trained observer | Improper PPE usage | PPE donning should be practiced and competency assessed, prior to conducting the procedure Don BSL3+ PPE accompanied by a trained observer Use a PPE donning checklist to ensure PPE is properly applied. Have the trained observer confirm that the PPE is in good condition and integrity is satisfactory | It is recommended to follow BSL-3+ PPE guidelines. At minimum PPE should include: Fit-tested N95 (or higher) respirator* Liquid impermeable closed front gown OR full body jumpsuit Long cuff primary gloves Regular cuff secondary gloves Eye goggles Shoe covers * If additional respiratory protection is needed, it’s recommended to use either a powered air purifying respirator (PAPR) or a controlled air purifying respirator (CAPR), with CAPRs being the most protective. Note that CAPRs and PAPRs will need to be disinfected after each use. Follow manufacturer’s guidelines for proper disinfection and maintenance |
| 4 | Preparation of specimens for testing | Aerosolization, splash or splatter | Minimize the number of workers handling the specimens Use BSL-3 PPE: Liquid-impermeable closed front gown OR full body jumpsuit double gloves N95 respirator and goggles or full-face shield (ensure eyes and mucous membranes covered) Work should be performed inside a certified class II BSC, following CDC’s acceptable BSC practices: Keep sash at the appropriate level Adjust chair to appropriate height for user Minimize unnecessary movements while working in the BSC Limit the traffic around the BSC Work over a moistened, plastic-backed absorbent pad with appropriate disinfectant Only use pipette tips with barrier filters Have a dedicated rigid waste container and sharps container in the BSC | Prepare disinfectant according to manufacturer’s instructions (e.g., EPA List L: Disinfectants for Use Against Ebola Virus) and wipe all materials down with the disinfectant before bringing into BSC No exposed skin inside the BSC Bring all necessary materials into the BSC before starting to work. Verify BSC is operating at acceptable flow rate Minimize use of sharps; dispose of all pipette tips and sharps in the dedicated containers in the BSC Specimens, equipment and all materials must be decontaminated before removing from BSC DO NOT set up any viral cultures Put still-sealed transfer pipets in the BSC, as contamination with bleach or nucleic acids can equate to false negatives or positives |
| 5 | Sample testing | Accidental transfer of contaminated material from the BSC, resulting in an exposure | Do not touch the inside of caps, open mouths or the buffer ampule’s nozzle to prevent contamination. Clean loading block with appropriate disinfectant capable of destroying nucleic acid and disinfecting | Use 10% bleach disinfectant followed by appropriate contact time; remove residue with DI water or 70% ethanol. Change outer gloves within the BSC and dispose in waste receptacle within BSC Place buffer ampule in middle of block with end hanging off the back to prevent contamination with uncleaned nucleic acid or unremoved bleach residue |
|  |  | Sharps Hazard | Use hand placement as recommended by instrument manufactures guidelines | Suggested procedure for buffer handling: Hold the buffer tube with the nozzle pointed up Pinch at break point to crush open, using thumb and index finger. Rotate buffer tube 90 degrees, still with the nozzle pointed up, and break again. Invert buffer ampule over sample tube without touching it Dispense buffer using one continuous pinch from top to bottom of ampule using both hands’ thumbs and index fingers, stopping when bubbles form The hydration solution may puncture PPE and, if unnoticed, lead to chance for biohazard exposure |
|  |  | Aerosolization, splash or splatter | Process is enclosed within the unit (although not airtight) Use proper transfer pipet technique | See Sharps Hazard Additional Information for correct buffer handling procedure Liquid may dispense while transferring between the sample and reagent tubes Discard spent pipet as biohazard waste in leakproof container Close reagent tube with thumb; ensure it is fully closed, as loose tops can pop open and expel not-yet-inactivated sample/buffer mixture Keep thumb on closed cap when inverting Place sample tube back in its holder, twist to release the seal, and wait five (5) seconds; not waiting can cause dripping when inserting the sample tube needle into the pouch’s port |
| 6 | After processed specimen inactivation and before removal of submitted specimen from BSC | Accidental transfer of contaminated material from the BSC | Wipe all tubes with disinfectant before removing from BSC Place remaining specimen in Ziploc plastic bag; disinfect exterior of bag before removing from BSC. Dispose of primary gloves in waste receptacle within BSC Store specimen(s) in a secure refrigerator inside BSL-3 suite, with limited access | Use appropriate disinfectants for containers and work surfaces A dedicated Category A waste bag for all waste generated is recommended during specimen processing |
| 7 | Post extraction BSC decontamination | Contamination of BSC surfaces, leading to the possibility of future exposures | Wipe the inside of the BSC with approved disinfectant Remove all PPE and discard into biohazard waste container | Use 10% bleach disinfectant followed by appropriate contact time Wipe down all surfaces in the BSC with 70% alcohol and allow to air dry. |
| 8 | Waste autoclaving | External contamination of waste containers | Twist and secure (with tape) the opening of the biohazard waste bags Disinfect outside of waste containers with approved disinfectant before removal from BSC and BSL-3 | To clean exterior of the waste container, use 10% bleach disinfectant followed by appropriate contact time Autoclave all PPE and laboratory consumables used in specimen handling and testing. |
| 9 | Doff PPE with trained observer | Improper PPE removal can result in exposure | Doff BSL3+ PPE accompanied by a trained observer Use a PPE doffing checklist to ensure PPE was properly removed to prevent exposure. Follow your institutional hand washing policy Have the trained observer confirm that the individual doffing PPE has successfully avoided direct contact with contaminated areas and has no observable contamination on their skin or clothing | CDC’s Clinical Safety: Hand Hygiene for Healthcare Workers CDC’s Clean Hands in Healthcare Training PPE doffing should be practiced and competency assessed prior to conducting the procedure in a real-world situation |

# Biological Safety

There are examples included in italics of what information might go in the response column, but this should be tailored to the laboratory’s procedures and inventory.

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| 1 | Biosafety level (BSL) established in this unit | ☐ N/A, no BSL |  |
|  |  | ☐ BSL-3+ | List BSL-3+ processes: e.g., Sample processing including inactivation and extraction steps; Sample storage |
|  |  | ☐ BSL-2 | List BSL-2 processes: e.g., PCR (after inactivation) and associated reagent preparation |
|  |  | ☐ BSL-1 | List BSL-1 processes: |
| 2 | List parts of testing process that pose a risk of aerosol generation | ☐ Vortexing |  |
|  |  | ☐ Specimen loading | e.g., Loading into Cartridge by syringe |
|  |  | ☐ Pipetting |  |
|  |  | ☐ Centrifuging |  |
|  |  | ☐ Opening and closing collection and microcentrifuge tubes |  |
| 3 | List all equipment involved in the use and storage of infectious materials Note: All must have biosafety labels affixed | ☐ BSC | ☐ has biosafety label |
|  |  | ☐ Centrifuge | ☐ has biosafety label |
|  |  | ☐ Freezers/refrigerators | ☐ has biosafety label e.g., Have 2 of each - 1 for specimen, 1 for reagents |
|  |  | ☐ Pipettes | ☐ has biosafety label e.g., Set is dedicated to Ebola testing |
|  |  | ☐ Heat block | ☐ has biosafety label |
| 4 | Are buckets with safety caps/cups or aerosol tight rotor lids used when centrifuging infectious materials? | ☐ No ☐ Yes |  |
| 5 | Is health monitoring performed in this Unit? | ☐ No ☐ Yes | If yes, please indicate frequency and the process: |
| 6 | Are vaccines recommended for work in this Unit? | ☐ No ☐ Yes | If yes, please indicate how employees are informed of the vaccines, what vaccines are recommended, etc.: |
| 7 | List all sharps used and whether they include any safety features | ☐ Safety blades | ☐ includes safety device feature |
|  |  | ☐ Syringe | ☐ includes safety device feature |
|  |  | ☐ Pipette tips | ☐ includes safety device feature |
|  |  | ☐ Reagent needles | ☐ includes safety device feature |
|  |  | ☐ No sharps used |  |
| 8 | Is a BSC in use? | ☐ No ☐ Yes | Please indicate if the BSC has been certified within the past year, whether the air vents are not blocked and the sash is in place and operable: |

## Comments

# Chemical Safety

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| ## Proper Labeling |  |  |  |
| 1 | All containers are labeled with chemical name | ☐ Yes ☐ No ☐ N/A |  |
| 2 | Fire Department Permit is posted on laboratory door | ☐ Yes ☐ No ☐ N/A |  |
| 3 | Chemical inventory is updated | ☐ Yes ☐ No ☐ N/A |  |
| 4 | Safety data sheets are accessible to staff | ☐ Yes ☐ No ☐ N/A |  |
| ## Chemical Storage |  |  |  |
| 5 | Incompatible chemicals are segregated | ☐ Yes ☐ No ☐ N/A |  |
| 6 | Flammable liquids are… |  |  |
|  | Stored in rated chemical cabinets | ☐ Yes ☐ No ☐ N/A |  |
|  | Stored in flammable-rated refrigerators/freezers | ☐ Yes ☐ No ☐ N/A |  |
| 7 | Excessive chemicals are stored in a chemical storage room | ☐ Yes ☐ No ☐ N/A |  |
| 8 | Compressed gas cylinders are stored in the laboratory | ☐ Yes ☐ No ☐ N/A |  |
| 9 | Chemicals are stored at eye-level | ☐ Yes ☐ No ☐ N/A |  |
| 10 | Acids and bases are… |  |  |
|  | Stored in a cabinet | ☐ Yes ☐ No ☐ N/A |  |
|  | Stored in a labeled area | ☐ Yes ☐ No ☐ N/A |  |
|  | Stored free from metals | ☐ Yes ☐ No ☐ N/A |  |
| ## Chemical Fume Hoods |  |  |  |
| 11 | Hoods have been certified within the past year | ☐ Yes ☐ No ☐ N/A |  |
|  | Sash is closed when not in use | ☐ Yes ☐ No ☐ N/A |  |
|  | Exhaust air is not blocked by large equipment or containers | ☐ Yes ☐ No ☐ N/A |  |
|  | Hoods are used for hazardous, toxic or flammable procedures | ☐ Yes ☐ No ☐ N/A |  |

## Additional Comments

# Personal Protective Equipment (PPE)

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| ## Education and Training |  |  |  |
| 1 | Laboratory staff are aware of PPE requirements for this procedure | ☐ Yes ☐ No |  |
| 2 | Staff receive an annual PPE competency assessment | ☐ Yes ☐ No |  |
| ## PPE Care |  |  |  |
| 3 | PPE are appropriately stored in laboratory | ☐ Yes ☐ No |  |
| 4 | PPE are inspected prior to use and are in good condition | ☐ Yes ☐ No |  |
| 5 | PPE are not worn outside the laboratory area | ☐ Yes ☐ No |  |
| ## PPE in Use |  |  |  |
| 6 | Safety glasses/goggles | ☐ Yes ☐ No |  |
| 7 | Facial shields/splash guards | ☐ Yes ☐ No |  |
| 8 | Disposable laboratory coats | ☐ Yes ☐ No |  |
| 9 | Nitrile gloves | ☐ Yes ☐ No |  |
| 10 | Respiratory protection | ☐ Yes ☐ No |  |
|  | If yes, users are enrolled in a respiratory protection program | ☐ Yes ☐ No |  |
| 11 | Safety glasses/goggles | ☐ Yes ☐ No |  |
| 12 | Facial shields/splash guards | ☐ Yes ☐ No |  |
| 13 | Cryo or autoclave gloves | ☐ Yes ☐ No |  |
| 14 | Over sleeves/booties/bonnet | ☐ Yes ☐ No |  |
| 15 | Closed-toe shoes that cover entire foot are worn in laboratory | ☐ Yes ☐ No |  |
| 16 | Proper Handwashing Technique | ☐ Yes ☐ No |  |

## Additional Comments

# Emergency Preparedness

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| 1 | Emergency contact information is posted | ☐ Yes ☐ No |  |
| 2 | First aid kit is maintained | ☐ Yes ☐ No |  |
| 3 | Biological spill kit is maintained | ☐ Yes ☐ No |  |
| 4 | Staff are aware of occupational injury procedures | ☐ Yes ☐ No |  |

## Additional Comments

# Documentation and Training

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| 1 | Employee(s) completed right-to-know training | ☐ Yes ☐ No |  |
| 2 | Employee(s) completed unit-specific training | ☐ Yes ☐ No |  |
| 3 | Employee(s) completed bloodborne pathogens training | ☐ Yes ☐ No |  |
| 4 | Employee(s) read and understand safety and health plans | ☐ Yes ☐ No |  |
| 5 | Employee has read the applicable procedural SOPs | ☐ Yes ☐ No |  |
| 6 | Employee has been assessed to conduct the applicable procedure safely and competently | ☐ Yes ☐ No |  |
| 7 | Door sign is up-to-date and posted | ☐ Yes ☐ No |  |
| 8 | Laboratory microwaves and refrigerators are labeled with “Not for Food or Drink – Biohazard” | ☐ Yes ☐ No |  |

## Additional Comments

# Waste Management

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| ## Chemical Waste |  |  |  |
| 1 | Containers are labeled with chemical name and percent of each chemical | ☐ Yes ☐ No |  |
| 2 | Containers are properly sealed | ☐ Yes ☐ No |  |
| 3 | Containers are in good condition for transport | ☐ Yes ☐ No |  |
| ## Biohazard Waste |  |  |  |
| 4 | Broken glass is placed in the appropriate receptacle | ☐ Yes ☐ No |  |

## Additional Comments

# Engineering Controls

| # | Item | Response | Comments |
| --- | --- | --- | --- |
| 1 | Biosafety cabinet | ☐ Yes ☐ No |  |
| 2 | Negative air pressure | ☐ Yes ☐ No |  |
| 3 | Transport containers | ☐ Yes ☐ No |  |
| 4 | Centrifuge rotor cups | ☐ Yes ☐ No |  |
| 5 | Sharps container | ☐ Yes ☐ No |  |

## Additional Comments

# At Risk Employees

| Name | Signature | Date |
| --- | --- | --- |

# Assessment Approval

This risk assessment should be reviewed annually or after any major changes (e.g., new facility, new employees, new technology, new method, changes in information for organism/agent, etc.). Minor changes should be recorded under Amendments. Major changes require a new risk assessment to be performed.

Reviews have been carried out on the following dates:

| Title | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| Principal Investigator |  |  |  |
| RO/ARO |  |  |  |
| Section Manager |  |  |  |
| Unit Manager |  |  |  |

Reviewed by:

| Testing Personnel | Printed Name | Signature | Date |
| --- | --- | --- | --- |

# LABORATORY SPECIMEN(S) HANDLING LOG

Use one form for each patient—multiple specimens per patient may use single log form.

Receiver of Specimen(s):

Date Received:  							Time:  					Intake #:

Delivery Method:    ☐ FedEx/UPS	☐ Storeroom	☐ Department of Health Driver	☐ Messenger/Name:

Laboratory Number Call for Pick-up:  							Time Picked Up by Laboratory:

# To be completed by the laboratory:

Picked up from Receiver By:

Purpose:   ☐ Patient Clinical Test	☐ QC	☐ Training/Validation 	☐ 		  Proficiency

Number of Specimens in Package: 			 	  	Sample Type(s):

Accession #: (1) 		 				 	(2) If Applicable:

Patient ID # on Submission Form: 		 		 	Submitter:

| Function | Responsible Technologist | Signature/ Initials of Responsible Technologist | Date and Time |
| --- | --- | --- | --- |
| Delivered to Lab/BSL-3 |  |  |  |
| Accessioned By |  |  |  |
| BSL-3 Processing |  |  |  |
| RNA Extraction |  |  |  |
| PCR Reaction Setup/Master Mix/Template |  |  |  |
| PCR Run |  |  |  |
| PCR Analysis |  |  |  |
| Storage of Original Sample(s) Unit: ______ Location: ______ |  |  |  |
| Storage of RNA Unit: Location: |  |  |  |
| LIMS Result Verification and Report Verification of Report Received ☐ |  |  |  |
| Results Reported To: ☐ CDC ☐ ______ |  |  |  |
| Specimen(s) Shipped To: ☐ CDC ☐ ______ |  |  |  |
| Other: |  |  |  |